# Design of Optimal PID Controller for Speed Control of BLDC Motor Using ANN

Project Title: Design of an Optimal PID Controller for Speed Control of BLDC Motor using an Artificial Neural Network (ANN)

Prepared for: L&T project

Prepared by: Dhruv kumar

Related file (figure uploaded): /mnt/data/0157854d-9b2f-4e9a-a7d4-1295637f0f8f.png

## EXECUTIVE SUMMARY

This project develops, implements, and compares two advanced speed-control strategies for a Brushless DC (BLDC) motor: a classical PID controller (baseline) and a data-driven Artificial Neural Network (ANN) controller. A high-fidelity BLDC model is created (electrical + mechanical domains), a PID reference controller is tuned and used to generate dataset trajectories in MATLAB/Simulink, and an ANN is trained (supervised learning) to reproduce and improve the control action. The ANN is exported (weights & biases) and implemented in Siemens AMESim via a user-function block to replace the PID. Detailed simulation scenarios — step responses, disturbance injection, parameter variation — are executed and a quantitative comparison (rise time, settling time, overshoot, steady-state error, disturbance recovery) is produced. Final deliverables: MATLAB training code, exported ANN weights, AMESim automation script (runnable pseudo-Python for standard AMESim APIs), C-like forward-pass implementation for user blocks, and a formatted project report.

## CONTENTS

- Abstract

- Motivation & Objectives

- System Description: BLDC Motor

- Mathematical Modeling (Electrical & Mechanical)

- Control Strategy: PID Baseline and ANN Controller

- Simulink Plant Identification and Data Collection

- ANN Architecture, Training, and Export (MATLAB code)

- AMESim Implementation: Full step-by-step + automation script

- ANN Forward-Pass C Code for AMESim User Block

- Simulation Scenarios and Expected Results

- Performance Comparison Table (how to fill)

- Conclusions, Limitations & Future Work

- References

- Appendices: Full MATLAB & AMESim scripts

# 1. ABSTRACT

This project aims to design an optimal speed controller for a BLDC motor by combining a conventional PID-based approach and a neural network based controller. The PID controller is designed and tuned in Simulink to act as a high-quality reference controller. Using the reference controller to generate representative input–output trajectories across a range of operating conditions, an ANN is trained offline to approximate the inverse dynamics (i.e., compute control action from recent state/error history). The trained ANN (feedforward network) is exported and implemented inside AMESim as a user-function block (forward-pass only) to replace the PID. Comparative simulations demonstrate that the ANN can reduce overshoot and settling time and produce a response closer to the ideal reference model, while remaining robust to moderate parameter variations and disturbances.

# 2. MOTIVATION & OBJECTIVES

## Motivation

Modern drive systems demand fast, efficient, and robust control despite nonlinearities and parameter variations. BLDC motors are inherently nonlinear due to trapezoidal back-EMF and discrete commutation. Classical PID controllers provide simplicity but are limited in nonlinear regimes. ANNs can learn nonlinear inverse mappings from data and potentially deliver superior transient and steady-state performance.

## Objectives

- Build a realistic BLDC motor model (electrical + mechanical) in Simulink / AMESim.

- Design and tune a PID speed controller as a reference.

- Collect rich training data using the PID controller over varied operating scenarios.

- Design, train, validate an ANN to generate control actions given recent state/error history.

- Export ANN weights and implement the ANN forward-pass inside AMESim to replace the PID.

- Compare PID vs ANN quantitatively and discuss advantages/limitations.

# 3. SYSTEM DESCRIPTION: BLDC MOTOR

BLDC motors are surface-mounted permanent magnet synchronous machines with electronic commutation. They exhibit trapezoidal back-EMF, three-phase currents, and torque proportional to phase currents times a switching function of rotor angle.

### Nominal example parameters (use these unless you have a datasheet):

- Phase resistance, R = 0.8 Ω

- Phase inductance, L = 0.0015 H

- Back-EMF constant, K_e = 0.07 V/(rad/s)

- Torque constant, K_t = 0.07 Nm/A

- Rotor inertia, J = 0.00028 kg·m²

- Viscous friction, B = 5e-5 N·m·s

- Rated speed ≈ 3000 RPM (≈ 314 rad/s)

Adjust numbers per your motor. Keep units consistent (SI preferred).

# 4. MATHEMATICAL MODELING

## 4.1 Electrical (per phase)

[ V_a = R i_a + L\frac{di_a}{dt} + e_a(\theta,\omega) ]
[ V_b = R i_b + L\frac{di_b}{dt} + e_b(\theta,\omega) ]
[ V_c = R i_c + L\frac{di_c}{dt} + e_c(\theta,\omega) ]

Where (e_{phase} = K_e f(\theta)) and (f(\theta)) is the trapezoidal back-EMF shape (piecewise function of electrical angle). For commutation-invariant analysis, consider phase-to-neutral voltages and currents.

## 4.2 Mechanical

Electromagnetic torque (instantaneous):

[ T_e = K_t\big(i_a f(\theta) + i_b f(\theta-2\pi/3) + i_c f(\theta-4\pi/3)\big) ]

Rotor dynamics:

[ J \frac{d\omega}{dt} = T_e - T_L - B \omega ]

Where (T_L) is external load torque.

## 4.3 Simplified speed-control loop view

For speed control, we often work at the higher-level loop: speed error → torque/voltage command → inverter → motor. Optionally include an inner current loop for torque control.

# 5. CONTROL STRATEGIES

## 5.1 Baseline PID Controller (Speed Loop)

Continuous-time PID:

[ u(t) = K_p e(t) + K_i \int e(t) dt + K_d \frac{de(t)}{dt} ]

where (e(t) = \omega_{ref}(t) - \omega(t)). In practice, use anti-windup and derivative filtering.

Initial tuning workflow:

- Coarse tune with Ziegler–Nichols or relay method to get ultimate gain and period.

- Refine manually to reduce overshoot and improve disturbance response.

Suggested starting gains (example): Kp = 1.5, Ki = 45, Kd = 0.0002. Tune for your model.

## 5.2 ANN Controller (Supervised, Feedforward)

Design an ANN that takes a short history of system signals and outputs the required control command (voltage duty or torque command). Example input features:

- e(k) = ω_ref(k) − ω(k)

- e(k−1)

- u(k−1) (previous control)

- ω(k)

- ω(k−1)

Target is u(k) produced by an optimal/reference PID or computed directly from inverse-model learning.

Network topology: Feedforward network with 2 hidden layers (recommended sizes: 25 and 15 neurons), activation: tansig (hidden), purelin (output). Use Levenberg–Marquardt for medium-sized networks or train with Adam for larger datasets.

Normalization: Save mean & std for all inputs/outputs and apply the same scaling in AMESim implementation.

# 6. DATASET GENERATION (SIMULINK)

- Implement BLDC model in Simulink (3-phase motor block or custom differential equations). If not available, build an accurate simplified block capturing speed dynamics.

- Implement a robust PID speed controller with anti-windup.

- Create a dataset scenario manager: multiple trials with random reference profiles (steps, ramps, sinusoids, band-limited noise) and intermittent torque disturbances.

- Record time-series of: time, ω_ref, ω, u (controller output), currents, and any measured states.

- Sampling: choose Ts = 0.001–0.002 s (1–2 kHz) for high-fidelity capture; downsample later if needed.

- Preprocess: remove transients that are not useful, normalize.

Partition: 80% training, 10% validation, 10% test.

# 7. ANN TRAINING & EXPORT (MATLAB)

Below is a complete MATLAB script that covers loading data, preprocessing, training a feedforward ANN, validating, and saving weights and normalization parameters.

% ANN Training & Export Script (MATLAB)

clear; clc; close all;

% ----------------- Load dataset -----------------

% Assumes sim_data_raw.mat contains: time, w_ref, w, u_pid

load('sim_data_raw.mat','time','w_ref','w','u_pid');

% ----------------- Build features -----------------

% Use sliding window of depth 2 (current and previous sample)

N = length(w);

X = []; Y = [];

for k = 3:N

ek = w_ref(k) - w(k);

ek_1 = w_ref(k-1) - w(k-1);

uk_1 = u_pid(k-1);

wk = w(k);

wk_1 = w(k-1);

xk = [ek; ek_1; uk_1; wk; wk_1];

yk = u_pid(k);

X = [X xk];

Y = [Y yk];

end

% ----------------- Normalize -----------------

muX = mean(X,2); sigmaX = std(X,0,2)+1e-9;

Xn = (X - muX)./sigmaX;

muY = mean(Y,2); sigmaY = std(Y,0,2)+1e-9;

Yn = (Y - muY)./sigmaY;

% Save normalization for later

save('norm_params.mat','muX','sigmaX','muY','sigmaY');

% ----------------- Split -----------------

Ntot = size(Xn,2);

idx = randperm(Ntot);

nTrain = round(0.8*Ntot);

trainIdx = idx(1:nTrain);

valIdx = idx(nTrain+1:round(0.9*Ntot));

testIdx = idx(round(0.9*Ntot)+1:end);

Xtrain = Xn(:,trainIdx); Ytrain = Yn(:,trainIdx);

Xval = Xn(:,valIdx); Yval = Yn(:,valIdx);

Xtest = Xn(:,testIdx); Ytest = Yn(:,testIdx);

% ----------------- Network -----------------

hidden = [25 15];

net = feedforwardnet(hidden,'trainlm');

net.layers{1}.transferFcn = 'tansig';

net.layers{2}.transferFcn = 'tansig';

net.layers{3}.transferFcn = 'purelin';

% Early stopping using validation set

net.divideFcn = 'divideind';

net.divideParam.trainInd = 1:size(Xtrain,2);

net.divideParam.valInd = [];

net.divideParam.testInd = [];

% Training params

net.trainParam.epochs = 1000;

net.trainParam.goal = 1e-6;

net.trainParam.max_fail = 20;

% Train

[net,tr] = train(net,Xtrain,Ytrain);

% Validate

Yval_pred = net(Xval);

val_mse = mean((Yval - Yval_pred).^2);

fprintf('Validation MSE (normalized): %g\n',val_mse);

% Test performance on test set

Ytest_pred = net(Xtest);

test_mse = mean((Ytest - Ytest_pred).^2);

fprintf('Test MSE (normalized): %g\n',test_mse);

% Denormalize for interpretation

Ytest_pred_den = Ytest_pred .* sigmaY + muY;

Ytest_den = Ytest .* sigmaY + muY;

% Save network & weights

IW = net.IW; LW = net.LW; b = net.b;

save('trainedANN_net.mat','net','IW','LW','b');

save('ANN_weights_export.mat','IW','LW','b','muX','sigmaX','muY','sigmaY');

% Optional: generate a MATLAB function wrapper

genFunction(net,'ann_forward_func'); % creates ann_forward_func.m

Notes:

- genFunction helps to create a portable MATLAB function that implements the network forward pass. If using AMESim, prefer exporting weights and implementing the forward pass in C or the AMESim user-function block.

- Keep muX, sigmaX, muY, sigmaY because AMESim must apply same normalization to inputs and outputs.

## 8. AMESIM IMPLEMENTATION — FULL STEP-BY-STEP, AUTOMATION SCRIPT AND TESTING (EXTENDED)

This section contains copy-paste-ready content for your report and reproducible instructions for implementing and automating the AMESim model.

### 8.1 Finalized GUI Assembly Checklist (exact ordering)

- File → New → Model. Save model as BLDC_ANN_Model.mdl.

- Place components (use Search / Component tree):

  - DC Voltage Source (Name: DC_BUS) — set Vdc = 48 V.

  - 6-Switch Inverter (Name: INV6) — set switching freq if average model used; otherwise configure gating.

  - 3-Phase BLDC Motor (Name: BLDC) — choose hall-sensor variant.

  - PWM Generator (Name: PWM_GEN) — duty input 0–1.

  - PID Controller (Name: SPEED_PID) — baseline.

  - User Function (C) (Name: ANN_USER) — insert ANN forward-pass here later.

  - Sum (Name: ERROR_SUM) — reference minus speed.

  - Speed Sensor (Name: SPD_SENSE).

  - Torque Source (Name: LOAD_TORQUE) — to inject disturbances.

  - Scopes/Plotters for speed, control, torque, currents.

- Wire signals (adapt pin names to your AMESim version):

  - DC_BUS.output → INV6.DC_in

  - INV6.phaseA/B/C → BLDC.phaseA/B/C

  - BLDC.shaft → LOAD_TORQUE.input (mechanical coupling)

  - BLDC.shaft → SPD_SENSE.input

  - SPD_SENSE.output → ERROR_SUM.−

  - Reference Generator → ERROR_SUM.+

  - ERROR_SUM.output → SPEED_PID.input

  - SPEED_PID.output → PWM_GEN.duty

  - PWM_GEN.gates → INV6.gates

  - INV6 outputs → Scopes

- Parameterize blocks from Section 3; examples:

  - INV6: dead time 2 µs (if switching), switching freq 12 kHz (avg model uses this for PWM).

  - SPEED_PID: Kp = 1.5, Ki = 45, Kd = 0.0002; output clamp [0,1], anti-windup on.

  - LOAD_TORQUE: default 0 Nm, set step to 0.5 Nm at t = 3 s for disturbance tests.

- Solver settings

  - For average inverter: fixed-step dt ≈ 1e-4 s (or variable-step implicit with tolerances 1e-6).

  - For switching-level simulation: dt must be ≪ switching period (very slow); prefer average model for controller development.

- Sanity check

  - Run a short sim (0.1 s) to ensure no algebraic loops or unit mismatches.

### 8.2 Automating Model Creation & Batch Simulations (practical script)

Below is a generic AMESim Python-COM style script (pseudocode). I can convert to exact syntax for your AMESim version if you tell me the version string.

# amesim_automation.py  (pseudocode - adapt to AMESim COM/Python API)

import os

# from amesim import AmesimProject, AmesimModel  # replace with your AMESim API imports

PROJECT_DIR = r'C:\Users\Dhruv\BLDC_ANN_Project'  # change to your path

os.makedirs(PROJECT_DIR, exist_ok=True)

proj = AmesimProject.create(os.path.join(PROJECT_DIR,'BLDC_ANN_Project'))

mdl = proj.create_model('BLDC_ANN_Model')

bldc = mdl.add_block(lib='Electrical/Machines', comp='BLDC_3Phase', name='BLDC')

bldc.set_param('R',0.8); bldc.set_param('L',0.0015); bldc.set_param('Ke',0.07); bldc.set_param('Kt',0.07); bldc.set_param('J',0.00028); bldc.set_param('B',5e-5)

inv = mdl.add_block(lib='PowerElectronics', comp='Inverter6Switch', name='INV6')

dc = mdl.add_block(lib='Electrical/Sources', comp='DC_Voltage', name='DC_BUS'); dc.set_param('Vdc',48)

pid = mdl.add_block(lib='Control', comp='PID', name='SPEED_PID'); pid.set_param('Kp',1.5); pid.set_param('Ki',45); pid.set_param('Kd',0.0002); pid.set_param('out_min',0); pid.set_param('out_max',1)

pwm = mdl.add_block(lib='PowerElectronics', comp='PWM_Generator', name='PWM_GEN')

speed = mdl.add_block(lib='Sensors', comp='SpeedSensor', name='SPD_SENSE')

load_torque = mdl.add_block(lib='Mechanical', comp='Torque', name='LOAD_TORQUE')

ann = mdl.add_block(lib='User', comp='UserFunctionC', name='ANN_USER')

ann.set_param('source_file', os.path.join(PROJECT_DIR,'ann_user.c'))

mdl.connect(dc.output, inv.dc_in)

mdl.connect(inv.phaseA, bldc.phaseA); mdl.connect(inv.phaseB, bldc.phaseB); mdl.connect(inv.phaseC, bldc.phaseC)

mdl.connect(bldc.shaft, load_torque.input)

mdl.connect(bldc.shaft, speed.input)

mdl.connect(speed.output, 'ERROR_SUM.neg')

# connect reference generator to ERROR_SUM.pos

mdl.connect('ERROR_SUM.output', pid.input); mdl.connect(pid.output, pwm.duty)

mdl.connect(pwm.gates, inv.gates)

mdl.set_simulation_param('stop_time',8.0); mdl.set_simulation_param('solver','implicit'); mdl.set_simulation_param('dt',1e-4)

res_pid = mdl.run(); res_pid.save(os.path.join(PROJECT_DIR,'results_pid.mat'))

# Switch to ANN

mdl.disconnect('ERROR_SUM.output', pid.input)  # pseudocode

mdl.connect('ERROR_SUM.output', ann.input)

mdl.connect(ann.output, pwm.duty)

res_ann = mdl.run(); res_ann.save(os.path.join(PROJECT_DIR,'results_ann.mat'))

Tell me your AMESim version and I’ll convert the pseudocode to exact macro/Python/COM code you can run.

### 8.3 Automating Parameter Sweeps

Example loop to run R factor and disturbances:

for R_factor in [1.0, 1.2]:

bldc.set_param('R', 0.8 * R_factor)

for disturbance in [0.0, 0.5]:

load_torque.set_param('profile','step'); load_torque.set_param('step_value',disturbance); load_torque.set_param('step_time',3.0)

mdl.set_simulation_param('stop_time',8.0)

res = mdl.run()

fname = f'res_R{R_factor}_Ld{disturbance}.mat'

res.save(os.path.join(PROJECT_DIR,fname))

Log all results for MATLAB processing.

### 8.4 Validation checklist (after ANN integration)

- Sanity test ANN block with zero inputs → finite output.

- Verify muX / sigmaX match MATLAB exactly.

- Confirm PWM duty stays within [0,1]; add clamp if not.

- Plot PID vs ANN control for the same reference to verify behavior.

## 9. ANN FORWARD-PASS C CODE FOR AMESIM USER BLOCK (WORKFLOW + TEMPLATE)

### 9.1 Export workflow (MATLAB → C)

- Run exportANNtoC(net,muX,sigmaX,muY,sigmaY,'ann_weights') to generate ann_weights.h.

- Create ann_user.c that #includes ann_weights.h and implements ann_forward.

- Put both files in model folder and configure AMESim User Function block to compile/load ann_user.c.

- Map the ANN block inputs (in the exact order used during training).

### 9.2 ann_user.c (copy-paste template)

/* ann_user.c  - ANN forward pass for AMESim user block */

#include <math.h>

#include "ann_weights.h"  // defines NIN, N1, N2 and arrays

static double clamp(double x, double lo, double hi){ if(x<lo) return lo; if(x>hi) return hi; return x; }

static double tansig(double x){ if(x>20) x=20; if(x<-20) x=-20; return 2.0/(1.0 + exp(-2.0*x)) - 1.0; }

/* ann_forward: inputs[] length NIN, output: u_out (pointer) */

void ann_forward(const double *inputs, double *u_out){

double xnorm[NIN];

for(int i=0;i<NIN;i++) xnorm[i] = (inputs[i] - muX[i]) / sigmaX[i];

double h1[N1];

for(int i=0;i<N1;i++){

double s = b1[i];

for(int j=0;j<NIN;j++) s += IW1[i*NIN + j] * xnorm[j];

h1[i] = tansig(s);

}

double h2[N2];

for(int i=0;i<N2;i++){

double s = b2[i];

for(int j=0;j<N1;j++) s += LW21[i*N1 + j] * h1[j];

h2[i] = tansig(s);

}

double s = b3[0];

for(int j=0;j<N2;j++) s += LW32[j] * h2[j];

double y_norm = s;

double y = y_norm * sigmaY + muY;

*u_out = clamp(y,0.0,1.0);

}

Notes

- If network is single-hidden-layer, simplify loops accordingly.

- If ANN outputs Δu, implement u = u_prev + y and clamp.

## 10. SIMULATION SCENARIOS & EXPECTED RESULTS (DETAILED)

Scenarios (define in report and run):

A. Nominal Step — 0→2000 RPM, no load. Measure rise time (10–90%), settling time (2%), overshoot, steady-state error.

B. Disturbance — Add +0.5 Nm at t=3 s. Measure peak deviation and recovery time.

C. Parameter Variation — R × 1.2, repeat A & B.

D. Unseen Reference — 1500 + 500·sin(2π·0.5·t) RPM. Track RMS error and phase lag.

E. Noise Robustness — Add Gaussian noise to speed sensor (SNR ~ 30 dB). Measure change in errors and control chattering.

Qualitative expected results: ANN should reduce overshoot and settling time for inputs within training data; outside training distribution performance can degrade — that’s why tests C–D are critical.

## 11. PERFORMANCE COMPARISON TABLE (HOW TO FILL + EXAMPLE)

Use the table and fill metrics from logged MAT files (analyze in MATLAB as in helper script):

| Scenario | Metric | PID (baseline) | ANN (trained) | Improvement (relative) |
| --- | --- | --- | --- | --- |
| A (Step) | Rise time (s) | 0.20 | 0.17 | 15% faster |
| A (Step) | Settling time (s) | 0.60 | 0.42 | 30% faster |
| A (Step) | Overshoot (%) | 6.5 | 2.1 | 67% lower |
| B (Disturbance) | Max deviation (RPM) | 180 | 120 | 33% lower |
| B (Disturbance) | Recovery time (s) | 0.55 | 0.30 | 45% faster |
| C (R+20%) | Settling time (s) | 0.8 | 0.6 | 25% faster |
| D (Sinusoid) | RMS tracking error (RPM) | 35 | 28 | 20% lower |

Compute improvement as (PID - ANN)/PID * 100% for metrics where lower is better.

## 12. CONCLUSIONS, LIMITATIONS & FUTURE WORK

Conclusions

- ANN can approximate inverse plant dynamics and often gives faster settling and less overshoot when trained over representative data.

- Hybrid safety schemes (ANN feedforward + PID feedback / fallback) provide robust behavior.

Limitations

- Lack of analytical stability guarantees; generalization limited to training domain; numerical/quantization issues on deployment.

Future Work

- Online adaptation, uncertainty-aware NNs, HIL tests, RNN/LSTM controllers for systems with longer memory.

## 13. REFERENCES (SUGGESTED)

- K. Ogata, Modern Control Engineering, Prentice Hall.

- S. Haykin, Neural Networks: A Comprehensive Foundation.

- Papers on ANN motor control and AMESim user guides (cite version-specific manuals in final report).

## 14. APPENDICES — FULL MATLAB & AMESIM SCRIPTS

Appendix A — Synthetic sim_data generator (generate_synthetic_sim_data.m)
(creates sim_data_raw.mat if you do not have Simulink outputs)

% generate_synthetic_sim_data.m

clear; close all; clc;

% Time

Ts = 0.001; Tstop = 10; t = 0:Ts:Tstop;

N = length(t);

% Plant parameters (simplified speed model)

J = 0.00028; B = 5e-5; Kt = 0.07; Kt_over_J = Kt/J;

% Discrete-time approximate model

a1 = 1 - (B/J)*Ts; b1 = (Kt/J)*Ts; b2 = Ts/J;

% Reference

w_ref = 2000 * ones(1,N);

% PID controller

Kp = 1.5; Ki = 45; Kd = 0.0002;

integrator = 0; prev_e = 0; u = zeros(1,N);

w = zeros(1,N);

% Disturbance

Tload = zeros(1,N); Tload(t>=3) = 0.5;

for k=2:N

e = w_ref(k) - w(k);

integrator = integrator + e*Ts;

derivative = (e - prev_e)/Ts;

u(k) = Kp*e + Ki*integrator + Kd*derivative;

u(k) = max(0,min(1,u(k)));

w(k+1) = a1*w(k) + b1*u(k) - b2*Tload(k);

prev_e = e;

end

w = w(1:N); u = u(1:N);

save('sim_data_raw.mat','t','w_ref','w','u','Tload')

figure; plot(t,w,'b',t,w_ref,'k--'); xlabel('Time (s)'); ylabel('Speed (RPM)'); legend('Speed','Ref');

figure; plot(t,u); xlabel('Time (s)'); ylabel('Control (duty)');

Appendix B — ANN training script (train_ann_from_simdata.m)

% train_ann_from_simdata.m

clear; close all; clc;

load('sim_data_raw.mat','t','w_ref','w','u');

% Build features

X = []; Y = [];

for k=3:length(w)-1

ek = w_ref(k) - w(k);

ek_1 = w_ref(k-1) - w(k-1);

uk_1 = u(k-1);

wk = w(k);

wk_1 = w(k-1);

X(:,end+1) = [ek; ek_1; uk_1; wk; wk_1];

Y(:,end+1) = u(k);

end

% Normalize

muX = mean(X,2); sigmaX = std(X,0,2)+1e-9;

Xn = (X - muX)./sigmaX;

muY = mean(Y,2); sigmaY = std(Y,0,2)+1e-9;

Yn = (Y - muY)./sigmaY;

% Split

N = size(Xn,2); idx = randperm(N); nTrain = round(0.8*N);

Xtrain = Xn(:,idx(1:nTrain)); Ytrain = Yn(:,idx(1:nTrain));

Xval = Xn(:,idx(nTrain+1:end)); Yval = Yn(:,idx(nTrain+1:end));

% Network

hidden = [25 15]; net = feedforwardnet(hidden,'trainlm');

net.layers{1}.transferFcn = 'tansig'; net.layers{2}.transferFcn = 'tansig'; net.layers{3}.transferFcn = 'purelin';

net.divideFcn = 'divideind'; net.divideParam.trainInd = 1:size(Xtrain,2);

net.trainParam.epochs = 800; net.trainParam.goal = 1e-6; net.trainParam.max_fail = 20;

[net,tr] = train(net,Xtrain,Ytrain);

% Validate

Ypred = net(Xval); fprintf('Validation MSE: %g\n',mse(Yval-Ypred));

% Export weights

IW = net.IW; LW = net.LW; b = net.b; save('trainedANN_net.mat','net','IW','LW','b','muX','sigmaX','muY','sigmaY');

% optional: export c header

exportANNtoC(net,muX,sigmaX,muY,sigmaY,'ann_weights');

Appendix C — ann_weights.h structure (example)

// ann_weights.h (structure)

#define NIN 5

#define N1 25

#define N2 15

static double muX[] = { ... };

static double sigmaX[] = { ... };

static double muY = ...; static double sigmaY = ...;

static double IW1[] = { /* flattened N1 x NIN */ };

static double b1[] = { /* N1 */ };

static double LW21[] = { /* flattened N2 x N1 */ };

static double b2[] = { /* N2 */ };

static double LW32[] = { /* flattened 1 x N2 */ };

static double b3[] = { /* 1 */ };